# Smart Water Tank Monitoring and Control System

## IFT 211 Mid Semester Lab Assessment

Student: Sam-Lebechukwu Ikenna
Matric No: 2025/A/SENG/0120
Date: July 17, 2026

# 1. Boolean Logic and K Maps

## Sensor Encoding

I used a priority encoder for the four water level inputs. The physical layout of a tank means that if the 100 percent sensor (S3) is wet, the lower sensors are also wet. So S3 gets the highest priority.

- Full: When S3 is 1, the level code is 11.

- Medium: When S3 is 0 but S2 is 1, the code is 10.

- Low: When S3 and S2 are 0 but S1 is 1, the code is 01.

- Empty: When only S0 is 1, the code is 00.

Based on the K maps for the two output bits, the simplified equations are:

- L1 = S3 + S2

- L0 = S3 + (NOT S2 AND S1)

## Pump Control

The tank is full when L1 and L0 are both 1. The pump has to stay on until it reaches that state.

- FULL = L1 AND L0

- PUMP = NOT FULL = NOT L1 OR NOT L0

## Overflow Counter and Alarm

To catch overflows, I added a 2 bit counter. It only counts up when the FULL signal is 1. Once it hits 3 (binary 11), it stops counting and activates the alarm. If FULL drops back to 0, the counter resets.

- Next state C1 = FULL AND (C1 OR C0)

- Next state C0 = FULL AND (C1 OR NOT C0)

- ALARM = C1 AND C0

# 2. Truth Tables

## Sensor Encoder

| S3 | S2 | S1 | S0 | L1 | L0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | X | 0 | 1 |
| 0 | 1 | X | X | 1 | 0 |
| 1 | X | X | X | 1 | 1 |

## Overflow Counter

| FULL | C1 | C0 | Next C1 | Next C0 | ALARM |
| --- | --- | --- | --- | --- | --- |
| 0 | X | X | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

# 3. Testing Scenarios

I ran the final Logisim file through five manual tests using the Poke tool.

- Empty state: I set S0 to 1 and the rest to 0. The display showed 0. The pump turned on and the alarm was off.

- Filling up: I turned on S1 and S2. The display updated to 2 after the next clock pulse. The pump kept running.

- Full capacity: I set S3 to 1. The display hit 3 and the pump stopped instantly. The overflow counter started.

- Triggering the alarm: I left S3 at 1 and pulsed the clock three more times. The counter hit 11, which turned the alarm light on. The pump stayed off.

- Draining the tank: I changed S3 back to 0. The pump kicked back on, the counter cleared to 00, and the alarm shut off.

# 4. System Explanation

The goal here was to automate a water tank using only logic gates. Four sensors check the water height and pass that data to a priority encoder, which turns it into a 2 bit binary number. I put a D flip flop register right after the encoder so the system stays synchronized with the clock.

From there, the 2 bit signal does a few different jobs. A comparator checks if the value is 11 and shuts the pump off if it is. A synchronous counter watches that same full signal and triggers an alarm if the tank stays full for three consecutive clock pulses. Finally, the binary level goes through a custom logic decoder to show the current water level as a number from 0 to 3 on the display.

The full design file for this circuit can be accessed at this Github repository.